# LO-SF-1G-ZX

1.25Gbps SFP 1550nm Optical Transceiver 80KM LC DDM

# Product Features

- SFP package with LC connector

- 1550nm DFP laser and APD photo detector

- Up to 120Km transmission on SMF

- +3.3V single power supply

- LVPECL compatible data input/output interface

- Low EMI and excellent ESD protection

- Laser safety standard IEC-60825 compliant

- Compatible with RoHS

# Applications

- 1.25Gb/s 1000Base-ZX Ethernet

Ordering information

| Part Number | Product Description |
| --- | --- |
| LO-SF-1G-ZX | SFP 1.25Gbps ZX+ 1550nm LC DDM SMF 80km |

| Parameter | Symbol | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| Storage Temperature | Tst | -40 | 85 | oC |
| Supply Voltage | Vcc | 0 | 3.6 | V |
| Operating Relative Humidity | RH | 5 | 95 | % |

Absolute Maximum Ratings

# Recommended Operating Conditions

| Parameter | Symbol | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 |  | 70 | oC |
| Supply Voltage | VCC | 3.15 | 3.3 | 3.45 | V |
| Data Rate | - | - | 1.25 | - | Gb/s |

# Transceiver Electrical Characteristics

| Transmitter Parameter |  | Symbol |  | Minimum |  | Typical |  | Maximum |  | Unit |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input Differential Impedance |  |  | ZIN |  | 90 |  | 100 |  | 110 |  | Ω |  | - |
| Differential Data Input Swing |  |  | VIN |  | 500 |  | - |  | 2400 |  | mV |  | - |
| TX_FAULT | Transmitter Fault |  | VOH |  | 2.0 |  | - |  | VCC |  | V |  | - |
|  | Normal Operation |  | VOL |  | 0 |  | - |  | 0.8 |  | V |  | - |
| TX_DISABLE | Transmitter Disable |  | VIH |  | 2.0 |  | - |  | VCC |  | V |  | - |
|  | Transmitter Enable |  | VIL |  | 0 |  | - |  | 0.8 |  | V |  | - |
| Receiver Parameter |  | Symbol |  | Minimum |  | Typical |  | Maximum |  | Unit |  | Notes |  |
| Output Differential Impedance |  |  | ZO |  | - |  | 100 |  | - |  | Ω |  | - |
| Differential Data Output Swing |  |  | VOUT |  | 370 |  | - |  | 2000 |  | mV |  | - |
| RX_LOS | Loss of signal (LOS) |  | VOH |  | 2.0 |  | - |  | VCC |  | V |  | - |
|  | Normal Operation |  | VOL |  | 0 |  | - |  | 0.8 |  | V |  | - |

# Transmitter Optical Characteristics

| Parameter | Symbol | Minimum | Typical | Maximum | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Center Wavelength Range | λc | 1540 | 1550 | 1560 | nm | - |
| Spectral Width (RMS) |  | - | - | 1 | nm | - |
| Extinction Ratio | ER | 8 | - | - | dB | - |
| Average Output Power | Po | 5 | - | -2 | dBm | - |
| Rise/Fall Time(20%~80%) | Tr/Tf | - | - | 180 | ns | - |
| Total jitter | Tj | - | - | 0.43 | Ul | - |
| Optical Eye Diagram | IEEE 802.3z and ANSI Fibre Channel Compatible |  |  |  |  |  |

# Receiver Optical Characteristics

| Parameter | Symbol | Minimum | Typical | Maximum | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Center Wavelength | λc | 1260 | - | 1620 | nm | - |
| Receiver Sensitivity | Rsen | - | - | -24 | dBm | - |
| Receiver Overload | Rov | -3 | - | - | dBm | - |
| Optical Return Loss | ORL | 12 | - | - | dB | - |
| LOS De-Assert | LOSD | - | - | -25 | dBm | - |
| LOS Assert | LOSA | -36 | - | - | dBm | - |
| LOS Hysteresis | - | 0.5 | - | 5 | - | - |

# Mechanical specifications